What Moves Us: Studies of Propulsion Measurement in Poststroke Walking

Andre Alvarez, MS

PhD in Rehabilitation Sciences

Dissertation Proposal

April 22nd, 2022

Contents

Background

Walking is an important function in human daily living; a large and increasing portion of adults in the United States walk either for leisure or transportation, and it has a meaningful impact on public health as an easily accessible form of moderate exercise than can reduce rates of chronic disease and medical expenditures1–3.  The efficiency of walking gait is important, with step-to-step transitions accounting for much of the energy cost4.  This energy is largely expended by the extended trailing limb in extension at the ankle by the ankle plantarflexors (PFs; the soleus, medial and lateral gastrocnemius), resulting in a propulsive force that moves the body forward.4–9. Both the magnitude and timing of this propulsion, measured as anterior-posterior ground reaction forces (APGRFs), are crucial to fast, efficient and stable walking4–6,10–12; mistimed and improper propulsion contributes to inefficiencies and increased energy costs5,13,14.

The impact of propulsion on walking gait is due to its effect on both the individual limb and the body’s center of mass (CoM)6. The first of these is explained by the large amount of mechanical energy being produced by the PFs: it is stored in the trailing limb as it enters the swing phase, with a minimal amount moving across the hip joint to the head, arms and trunk15. However, it’s been shown that the trailing limb works on the CoM during the step-to-step transition, increasing its speed and kinetic energy16. This is explained when it’s considered that, while the limb is a small portion of the total body mass, it contributes considerably to the dynamics of the body through its localized acceleration6.

A proposed equation by Zelik and Adamczyk helps to relay this point6. In it, the body is divided into two segments: the trailing limb (mass mlimb) and the remainder of the body (mass mROB). The position of the full body CoM (mass M = m limb+ mROB, position rROB) is defined by the position of the CoM of the two segment groups:

rCoM= rlimbmlimbM+ rROB(mROBM)

Taking consecutive time derivatives of CoM position produces CoM acceleration:

aCoM= alimbmlimbM+ aROB(mROBM)

As shown, the acceleration of the CoM (aCoM) is affected by both the accelerations of the trailing limb and the remainder of the body, in relation to their contribution to the total body mass (alimbmlimbM+ aROB(mROBM)). The remainder of the body would be the primary contributor to aCoM as it represents a much larger portion of the total body mass – unless alimb is sufficiently large when compared to aROB. This just so happens to be true in the case of human walking, where alimb is large and directed forward, and aROB is small and works in the opposite direction17. This shows that, with a large change in velocity, the trailing limb is a vital contributor to whole body energy change; this allows propulsion to be a major factor in the flow of the gait cycle5,18.

But what is a major factor can also be a hindrance. Inadequate propulsion from impaired control of paretic muscles such as the PFs can lead to inefficient step-to-step transitions. This is commonly seen in populations with neuromotor walking deficits, such as stroke. Stroke is a leading cause of disability that results in neuromotor impairments of walking and is one of the leading causes of long-term disability in the United States. It is estimated that 7 million Americans have had a stroke, with about 795,000 new or recurrent occurrences each year; it is projected that an additional 3.4 million people will have a stroke by 203019. A majority of poststroke individuals have a walking gait that is labor intensive and inefficient, walk at speeds that are not safe for walking in the community, and have much reduced physical activity; this leads to severe effects on their physical health and quality of life20–26.

One characteristic of walking poststroke is impaired propulsion function due to aging or neurological injury27–30. Inefficient walking patterns emerge from impaired PFs in an effort to redistribute positive work to the hip and knee joints; however, this distal-to-proximal reallocation of positive work does not offset the reduction in PF work 23,27,31. To compensate, strategies such as raising the trunk during pre-swing and swing, and moving the foot laterally during swing (hip hiking and circumduction, respectively) are often used to help provide floor clearance and reduce the risk of tripping29,32. This propulsion impairment results in up to 68% lower propulsion from the paretic limb and a reliance on the non-paretic limb for compensation23,28,33–36. Paretic propulsion is a well-documented variable in the poststroke population, with walking speed and long distance walking function both linked to community participation and perceived quality of life18,35,37–40.

While a primary goal of stroke rehabilitation is the return to efficient walking patterns, current therapies are insufficient in reducing this disability41–43. The priority of current rehabilitation protocols is to obtain walking independence as quickly as possible, often measured through walking speed32,44; while increases in walking speed are linked to walking independence, it is often the result of compensatory strategies29. Common solutions such as ankle-foot orthoses, while aiding in paretic limb ground clearance, have negative effects on propulsion and gait adaptability, leading to an increase in energy cost of walking45–47. In contrast, reducing gait asymmetries has shown to produce less costly walking48. While the development and study of interventions targeting propulsion function is an active area of research, there remains a need for measurement instruments that allow for the clinical management of individual limb propulsion deficits.

Specialized laboratory equipment such as force plates and optical motion tracking systems are the gold standard for measuring kinetics and kinematics in both healthy and neurologically impaired walking27,28,49–54. While their level of accuracy is unparalleled, these instruments present difficulties. First, several trials are needed for in-ground force plates to obtain an adequate amount of data as often only a few strides can be measured due to the number of force plates and inconsistent force strikes; this limits the number of consecutive gait cycles that can be reviewed. While instrumented treadmills solve the problem of limited gait cycles, they introduce the difficulty of unrealistically consistent gait speeds, as well as difference in overground and treadmill walking biomechanics. These instruments, along with optical motion capture, require large, dedicated spaces and are prohibitively expensive. Due to this, they are often found only in laboratory settings and are inaccessible to most clinicians.

Wearable sensors provide an opportunity to bridge this gap; they have been used in the study of gait biomechanics for decades and their use in the advancement of gait interventions is a highly active area of research55–59. Inertial sensors allow for the study of spatiotemporal (e.g. step length, cadence) and kinematic (e.g. joint angles) parameters. Combining accelerometers, gyroscopes and magnetometers, inertial measurement units (IMUs) are able to compensate for the drawbacks of each individual sensor; this allows for more robust analyses and computation of spatiotemporal parameters and kinematic variables such as joint angles59–62. Using kinematic data, IMUs can allow for the estimation of ground reaction forces – and, in turn, propulsion – using either biomechanical models or machine learning62–66.

Machine learning – programs that produce algorithms that adapt their outputs based on inputs to improve performance – is particularly of note67,68.  Supervised learning uses features (data taken from observation) to train a model on the correct responses; this allows the algorithm to generalize its response to all possible inputs67–69. This generalization is key, as the ability to produce reasonable outputs for inputs that were not encountered during training is useful for dealing with noise found in real-world data.

One example of supervised learning is regression, a statistical solution that aims to fit a mathematical function that passes as close as possible to all data points68. Linear models for regression are simple, and are often sufficient to gain insight into how the inputs affect the output in lower dimensional spaces69. Support vector machines (SVM) have been shown in the literature to be a useful supervised learning model for bipedal biomechanics70–73.

Many studies have shown that inertial sensors can produce data of comparable quality to the gold standard, even outside the laboratory74–86. With machine learning, IMUs may be a prime candidate to extending the study of paretic limb propulsion to the clinic.

Study 1: Timing and braking affect the relationship between paretic propulsion magnitude and long-distance walking function after stroke

Research Questions

The 6-Minute Walk Test (6MWT) is a popular outcome measure used to evaluate long distance walking function after stroke; however, the most often outcome measured, total distance walked, does not directly speak to the underlying neuromotor impairment. In order to promote individualized rehabilitation interventions that can target specific propulsion deficits, the question of what contributes biomechanically to reduced 6MWT performance is necessary. Biomechanical variables such as the peak propulsion force (the peak of the anterior ground reaction force) and the propulsion impulse (the delivered force of the anterior ground reaction force) have been shown to contribute to walking function after stroke35. However, these magnitude-based metrics do not account for temporal aspects of propulsion function, such as propulsion peak timing10; as they are propulsion-based, they also do not take into consideration braking magnitude and timing. There remains a need to determine if the timing and braking aspects of poststroke walking gait contribute to long-distance walking function.

Aim 1: Evaluate the effects of propulsion metrics, beyond peak propulsion magnitude and propulsion impulse, on long distance walking function

Hypothesis 1.1: Other propulsion metrics will contribute to the relationship of 6 Minute Walk Test performance, and peak propulsion and propulsion impulse, respectively

Hypothesis 1.2: Propulsion timing metrics will moderate the relationship between 6 Minute Walk Test performance, and peak propulsion and propulsion impulse, respectively

Methods

Participants

A subset of data collected on forty-five individuals with poststroke hemiparesis recruited for a study done at the University of Delaware35 were used in this study. Study inclusion criteria included: being at least six months post a single stroke; ability to walk at self-selected pace for 6 minutes with observable gait deficits and without orthotic support; ability to passively dorsiflex the ankle to a neutral position with the knee extended while in prone position; and passively extend the hip at least 10° while in a side lying position. Study exclusion criteria included: evidence of moderate to severe chronic white mater disease or cerebellar stroke on magnetic resonance imaging; history of lower extremity joint replacement; inability to communicate with investigators; neglect tested via the star cancellation test87 or hemianopia; score of >1 on Question 1b and >0 on Question 1c of the National Institutes of Health Stroke Scale; or unexplained dizziness in the last 6 months. All participants signed written informed consent forms approved by the Human Subjects Review Board of the University of Delaware and received medical clearance from their physician.

For this study, participants were excluded if they lacked a propulsion phase, defined as the crossing of the APGRF curve from a negative to positive value (i.e. from braking to propulsion – propulsion onset) near the middle of the stance phase, to the termination on contact. Additionally, a minimum of 10 strides of sufficient quality were necessary.

Data Collection

The 10-Meter Walk Test88 and the 6 Minute Walk Test89 were conducted by licensed physical therapists. A participant’s self-selected walking speed (i.e. comfortable walking speed, CWS) was derived from the 6-Meter Walk Test. The distance walked during the 6 Minute Walk Test was used as the measure of long-distance walking function, as it has been shown to accurately measure poststroke walking capacity and community ambulation40. The use of regular assistive device (e.g. cane) was allowed during testing, if necessary.

Kinetic data were collected during two separate bouts of 30 seconds of treadmill walking at their CWS, using a dual-belt instrumented treadmill with 2 independent 6-degree of freedom force platforms (Bertec Corporation, Worthington, OH) collecting ground reaction forces (GRFs) at 2100 Hz. Kinematic data was captured using an 8-camera motion capture system (Motion Analysis 3D Eagle, Santa Rose, CA) based on retroreflective markers placed on the pelvis, as well as bilaterally on the thigh, shank and foot segments. These included: the medial and lateral malleoli; the medial and lateral femoral condyles; the greater trochanters; and the iliac crests. All markers were sampled at a rate of 60 Hz.

Data Processing

Kinematic and kinetic data was processed in Visual3D and filtered using a bi-directional Butterworth low pass filter at 10 Hz. Using MATLAB 2019a (Mathworks, Natick, MA), the total number of slides per participant will be combined across both walking bouts. Each stride will be normalized to percent body weight (%bw) as per the literature, and percent stance time (%st, defined as the duration of the stance phase, from initial contact to final contact); the latter decision was made to better focus future analysis on the effects of propulsion, regardless of other factors in the gait cycle, as well as its common use by clinicians.

From this, a number of metrics will be calculated (Table 1). For reference, propulsion phase is defined as the crossing of the APGRF curve from a negative to positive value (i.e. from braking to propulsion – propulsion onset) near the middle of the stance phase, to the termination on contact; braking phase is defined from the initial contact to propulsion onset. Propulsion and braking impulse were calculated by obtaining the integral of points during the propulsion and braking phases, respectively.

| Metric | Definition | Units |
| --- | --- | --- |
| Propulsion Onset | Beginning of propulsion phase | %st |
| Propulsion Peak Magnitude | Maximum propulsion force | %bw |
| Propulsion Peak Timing | Timing of peak magnitude | %st |
| Propulsion Total Time | Time spent in propulsion phase | %st |
| Propulsion Impulse | Total propulsion force | - |
| Braking Peak Magnitude | Maximum braking force | %bw |
| Braking Peak Timing | Timing of peak braking | %st |
| Braking Total Time | Time spent in braking phase | %st |
| Braking Impulse | Total braking force | - |
| Time Between Peaks | Time between peak braking and peak propulsion magnitudes | %st |

Table 1: APGRF Metrics

Propulsion impulse and peak magnitude will be calculated as they have been shown in the literature to be correlated to long distance walking function35. However, these metrics do not consider the potential effects of timing on propulsion10; as such, propulsion peak timing and the total time of propulsion will be computed. Braking metrics may also help in explaining long distance walking function, as the ability of the paretic limb to slow the acceleration of the CoM may limit the propulsion of the non-paretic limb; in light of this, the braking metrics of peak magnitude, peak magnitude timing, impulse and total time of braking will be determined. Finally, the time between braking and propulsion peaks will also be calculated, as it may shed light on another aspect of propulsion timing, namely the transition between braking and propulsion phases.

Statistical Analysis

All statistical tests will be conducted in MATLAB. Residuals for all models will be screened for outliers. An alpha of 0.05 will be set as the threshold of significance. Changes in R2 and Radj2 will determine the viability of the created models; R2 uses the regression and total sum of squares to determine the variance between the model and a linear model, and Radj2 adds a penalty for the total number of observations, allowing for a better comparison between models with a different amount of variables.

To test the hypothesis that other propulsion metrics contribute to long-distance walking function, the ‘stepwisefit’ function will be used to perform stepwise linear regressions, using distance walked during the 6 Minute Walk Test as the response variable. As peak propulsion and propulsion impulse have been shown to contribute to walking function, they will be included in initial constant models; forward and backwards steps will be determined using a p-value tolerance of 0.05.

To test the hypothesis that propulsion timing metrics will moderate the relationship between long-distance walking function and propulsion magnitude metrics, the ‘stepwiselm’ function will be used to perform stepwise linear regressions including products of distinct predictors. Distance walked during the 6 Minute Walk Test will be used as the response variable. As peak propulsion and propulsion impulse have been shown to contribute to walking function, they will be included in initial constant models; forward and backwards steps will be determined using a p-value tolerance of 0.05.

Study 2: Validation of wearable sensors for use in estimating poststroke propulsion in the clinic

Research Questions

While the study and development of interventions targeting propulsion function after neurological injury is a highly active area of research18,74,75,90–96, the clinical use of these approaches is limited due to reduced access to the gold standard technology used to directly measure anterior-posterior ground reaction forces (APGRFs)23,27,28,36,51–54. As long as the instruments used to measure propulsion remain inaccessible to clinicians, interventions targeting specific deficits in walking function will continue to be out of reach.

Wearable sensors are a potential solution to this issue: they have been used to collect a number of gait measurements outside the laboratory74–78, and have proven effective at indirect measurements of ground reaction forces during walking79,80,97,98. Specifically, inertial measurement units (IMUs) have recently shown to be useful in measuring propulsion metrics, such as peak propulsion magnitude and timing, and propulsion impulse, in hemiparetic walking81,99. With the help of machine learning, IMUs may be the key to extending the study of paretic limb propulsion to the clinic.

Aim 1: Validate the accuracy of an IMU based algorithm in estimating propulsion metrics versus the gold standard

Hypothesis 1.1: This IMU based machine learning algorithm will accurately estimate all propulsion metrics against the gold standard

Hypothesis 1.2: This IMU based machine learning algorithm will outperform the published literature in estimating all propulsion metrics

Aim 2: Validate the accuracy of an IMU based algorithm in estimating the APGRF time series curve versus the gold standard

Hypothesis 2.1: This IMU based machine learning algorithm will accurately estimate the APGRF time series curve versus the gold standard

Hypothesis 2.2: This IMU based machine learning algorithm will outperform the published literature in estimating the APGRF time series curve

Methods

Participants

Data will be collected as part of an ongoing study in the Neuromotor Recovery Laboratory (NRL). A minimum of seven individuals with chronic poststroke hemiparesis will participate. Study inclusion criteria included: being greater than six months after stroke, ambulatory but with residual gait deficits, and having the ability to walk on a treadmill without orthotic support. Study exclusion criteria included: cerebellar stroke, lower extremity joint replacement or other orthopaedic conditions that change walking ability, pain that limits walking ability, inability to communicate with investigators, neglect or hemianopia, or unexplained dizziness, and more than two falls in the previous month. All study procedures will be approved by the Institutional Review Board Boston University. Written informed consent will be secured from all study participants prior to initiation of study procedures.

Data Collection

Participants will perform 10-Meter Walk Tests, conducted by licensed physical therapists, to obtain their self-selected walking speed (i.e. comfortable walking speed, CWS). Kinetic data will be collected during two separate 2-minute bouts of treadmill walking at 80% of their comfortable walking speed, as per study protocols. Ground reaction forces (GRFs) will be collected using a dual-belt instrumented treadmill with 2 independent 6-degree of freedom force platforms (Bertec Corporation, Worthington, OH) sampling at 2000 Hz.

Kinematic data will be captured by an 18-camera motion capture system (Qualisys, Göteborg, Sweden) based on the motion of reflective markers (Figure 1). Single ‘landmark’ markers will be placed to establish anatomy, including: the first and fifth metatarsals; the medial and lateral malleoli; the medial and lateral femoral condyles; the greater trochanters; the anterior superior iliac spines; and the iliac crests. Multiple ‘cluster’ markers will be placed to track the motion of the body segments, including: the pelvis, the upper legs (thighs), and the lower legs (shanks); two markers will also be placed on the heel to create a cluster with the metatarsal markers. All markers were sampled at a rate of 200 Hz.

Figure 1: Marker Placement

A total of 7 IMUs (Xsens Technologies B.V., Enschede, Netherlands), sampling at 100 Hz, will be collected concurrently with the motion capture data. They will be placed on the pelvis, as well as bilaterally on the thigh, shank and foot segments (Figure 2).

Figure 2: IMU Placement

Data Processing

Marker motion capture data will be clean in Qualysis Track Manager; marker and force plate data will be filtered in Visual3D  using a bi-directional Butterworth low pass filter at 30 Hz, then entered into MATLAB for further processing. Using MATLAB 2019a (Mathworks, Natick, MA), the total number of strides per participant will be combined across both walking bouts. Each stride will be normalized to percent body weight (%bw) as per the literature, and percent stance time (%st, defined as the duration of the stance phase, from initial contact to final contact); the latter decision was made to better focus future analysis on the effects of propulsion, regardless of other factors in the gait cycle, as well as its common use by clinicians. Propulsion metrics found to be significant as part of Study 1 will be calculated.

The IMU data will be cleaned and filtered in MATLAB, then conducted into features for use in the machine learning algorithm (Table 2). Univariate feature ranking will examine the importance of each predictor individually using an F-test, and then rank features using the p-values of the F-test statistics, an alpha of 0.05 determining inclusion in the model. Once obtained, these features will be used to train and test a supervised model using support vector machines (SVM), a common choice in the literature70–73; 70% of the data used for training and 30% for testing to determine the generalization of the model. This process will be completed for both the propulsion metrics, as well as for the APGRF time series curve.

| Source | Definition | Directions | Features |
| --- | --- | --- | --- |
| Accelerometer | Linear acceleration | X, Y, Z | 3 |
| Gyroscope | Angular velocity | X, Y, Z | 3 |
| Magnetometer | Strength of magnetic field | X, Y, Z | 3 |
|  |  | Total | 9 |

Table 2: Features from IMUs

For Aim 1, the inputs to the model will be the features listed in Table 2; the response variables will be the metrics calculated in Study 1 (Table 1), taken from the gold standard. Additionally, individual models will be trained to estimate each of the metrics found significant in Study 1, using the same inputs as the larger model. For Aim 2, the inputs to the model will be the features listed in Table 2; the response variable will be the APGRF time series curve taken from the gold standard.

Statistical Analysis

To determine the performance of the models, root mean squared error (RMSE) and R2 will be used.  RMSE measures the average of the squared difference between the predictions and the ground truth, allowing for the detection of outliers; R2 uses the regression and total sum of squares to determine the variance between the model and a linear. For propulsion metrics, the degree of absolute agreement among direct and indirect measurements will be evaluated using two-way mixed effect, absolute agreement single rater intraclass correlation coefficients (ICCs)100,101 with an alpha of 0.05.

To test the hypothesis that the IMU based algorithm will accurately estimate propulsion metrics against the gold standard, the features selected for this purpose will be used to train a SVM model, using propulsion metrics taken from the gold standard of force plate data as the response variable. Ideally, RMSE will be below 5%; R2 and ICC will both be above 0.9. The same model will then be compared to those in the literature, with performance metrics as well as features used to construct the models being analyzed. A discussion on the chosen features and their biomechanical relationship to the propulsion metrics will follow.

To test the hypothesis that the IMU based algorithm will accurately estimate the APGRF time series curve versus the gold standard, the features selected for this purpose will be used to train a SVM model, using the APGRF time series curve taken from the gold standard of force plate data as the response variable. Ideally, RMSE will be below 5%; R2 and ICC will both be above 0.9. The same model will then be compared to those in the literature, with performance metrics as well as features used to construct the models being analyzed. A discussion on the chosen features and their biomechanical relationship to the propulsion metrics will follow.

Limitations

There are inherent limitations to the proposed studies. As the population and walking conditions examined are of a specific kind, the results of these studies may not be suitable for consideration in other populations or in lower level walkers in the poststroke population. Also, when studying human movement with wearable sensors, drift in the component signals must be taken into consideration, and IMUs must be securely mounted to the body segments to minimize soft tissue artifacts and avoid the sensor moving relative to the body segment.

Bibliography

1. 	Berrigan D, Carroll DD, Fulton JE, et al. Vital signs: walking among adults-United States, 2005 and 2010. Morbidity and Mortality Weekly Report. 2012;61(31):595-601.

2. 	Kruger J, Ham SA, Berrigan D, Ballard-Barbash R. Prevalence of transportation and leisure walking among U.S. adults. Preventive Medicine. 2008;47(3):329-334. doi:10.1016/j.ypmed.2008.02.018

3. 	Lee IM, Buchner DM. The Importance of Walking to Public Health. Medicine & Science in Sports & Exercise. 2008;40(7):S512-S518. doi:10.1249/MSS.0b013e31817c65d0

4. 	Donelan JM, Kram R, Kuo AD. Mechanical work for step-to-step transitions is a major determinant of the metabolic cost of human walking. Journal of Experimental Biology. 2002;205(23):3717-3727. doi:10.1007/BF00430237

5. 	Kuo AD, Donelan JM. Dynamic Principles of Gait and Their Clinical Implications. In: Physical Therapy. Vol 90. Oxford University Press; 2010:157-174. doi:10.2522/ptj.20090125

6. 	Zelik KE, Adamczyk PG. A unified perspective on ankle push-off in human walking. The Journal of Experimental Biology. 2016;219(23):3676-3683. doi:10.1242/jeb.140376

7. 	Cappozzo A, Figura F, Marchetti M, Pedotti A. The interplay of muscular and external forces in human ambulation. Journal of biomechanics. 1976;9(1):35-43.

8. 	Hof AL, Nauta J, van der Knaap ER, Schallig MAA, Struwe DP. Calf muscle work and segment energy changes in human treadmill walking. Journal of Electromyography and Kinesiology. 1992;2(4):203-216. doi:10.1016/1050-6411(92)90024-D

9. 	Winter DA. Energy generation and absorption at the ankle and knee during fast, natural, and slow cadences. Clinical orthopaedics and related research. 1983;(175):147-154.

10. 	Kuhman D, Hurt CP. The timing of locomotor propulsion in healthy adults walking at multiple speeds. Human Movement Science. 2019;68:102524. doi:10.1016/j.humov.2019.102524

11. 	Riley PO, Croce UD, Casey Kerrigan D. Propulsive adaptation to changing gait speed. Journal of Biomechanics. 2001;34(2):197-202. doi:10.1016/S0021-9290(00)00174-3

12. 	Lewek MD, Randall EP. Reliability of Spatiotemporal Asymmetry During Overground Walking for Individuals Following Chronic Stroke. Journal of Neurologic Physical Therapy. 2011;35(3):116-121. doi:10.1097/NPT.0b013e318227fe70

13. 	Kramer S, Johnson L, Bernhardt J, Cumming T. Energy Expenditure and Cost During Walking After Stroke: A Systematic Review. Archives of Physical Medicine and Rehabilitation. 2016;97(4):619-632.e1. doi:10.1016/j.apmr.2015.11.007

14. 	Mian OS, Thom JM, Ardigò LP, Narici MV, Minetti AE. Metabolic cost, mechanical work, and efficiency during walking in young and older men. Acta Physiologica. 2006;186(2):127-139. doi:10.1111/j.1748-1716.2006.01522.x

15. 	Winter DA, Robertson DGE. Joit torque and energy patterns in normal gait. Biological cybernetics. 1978;29(3):137-142.

16. 	Donelan JM, Kram R, Kuo AD. Simultaneous positive and negative external mechanical work in human walking. Journal of Biomechanics. 2002;35(1):117-124. doi:10.1016/S0021-9290(01)00169-5

17. 	Lipfert S, Gunther M, Renjewski D, Seyfarth A. Impulsive ankle push-off powers leg swing in human walking. Journal of Experimental Biology. 2014;217(10):1831-1831. doi:10.1242/jeb.107391

18. 	Browne MG, Franz JR. Does dynamic stability govern propulsive force generation in human walking? Royal Society Open Science. 2017;4(11):171673. doi:10.1098/rsos.171673

19. 	Virani SS, Alonso A, Benjamin EJ, et al. Heart Disease and Stroke Statistics—2020 Update: A Report From the American Heart Association. American Heart Association; 2020:CIR. 0000000000000757.

20. 	Awad LN, Bae J, Kudzia P, et al. Reducing Circumduction and Hip Hiking During Hemiparetic Walking Through Targeted Assistance of the Paretic Limb Using a Soft Robotic Exosuit. American Journal of Physical Medicine & Rehabilitation. 2017;96(10):S157-S164. doi:10.1097/PHM.0000000000000800

21. 	Duncan PW, Sullivan KJ, Behrman AL, et al. Body-Weight–Supported Treadmill Rehabilitation after Stroke. New England Journal of Medicine. 2011;364(21):2026-2036. doi:10.1056/NEJMoa1010790

22. 	English C, Manns PJ, Tucak C, Bernhardt J. Physical Activity and Sedentary Behaviors in People With Stroke Living in the Community: A Systematic Review. Physical Therapy. 2014;94(2):185-196. doi:10.2522/ptj.20130175

23. 	Farris DJ, Hampton A, Lewek MD, Sawicki GS. Revisiting the mechanics and energetics of walking in individuals with chronic hemiparesis following stroke: From individual limbs to lower limb joints. Journal of NeuroEngineering and Rehabilitation. 2015;12(1):1-12. doi:10.1186/s12984-015-0012-x

24. 	Michael KM, Allen JK, Macko RF. Reduced Ambulatory Activity After Stroke: The Role of Balance, Gait, and Cardiovascular Fitness. Archives of Physical Medicine and Rehabilitation. 2005;86(8):1552-1556. doi:10.1016/j.apmr.2004.12.026

25. 	Rand D, Eng JJ, Tang PF, Hung C, Jeng JS. Daily physical activity and its contribution to the health-related quality of life of ambulatory individuals with chronic stroke. Health and Quality of Life Outcomes. 2010;8(1):80. doi:10.1186/1477-7525-8-80

26. 	Reisman DS, Rudolph KS, Farquhar WB. Influence of speed on walking economy poststroke. Neurorehabilitation and neural repair. 2009;23(6):529-534. doi:10.1177/1545968308328732

27. 	Bowden MG, Balasubramanian CK, Neptune RR, Kautz SA. Anterior-posterior ground reaction forces as a measure of paretic leg contribution in hemiparetic walking. Stroke. 2006;37(3):872-876. doi:10.1161/01.STR.0000204063.75779.8d

28. 	Jonkers I, Delp S, Patten C. Capacity to increase walking speed is limited by impaired hip and ankle power generation in lower functioning persons post-stroke. Gait & Posture. 2009;29(1):129-137. doi:10.1016/j.gaitpost.2008.07.010

29. 	Chen G, Patten C, Kothari DH, Zajac FE. Gait differences between individuals with post-stroke hemiparesis and non-disabled controls at matched speeds. Gait & Posture. 2005;22(1):51-56. doi:10.1016/j.gaitpost.2004.06.009

30. 	Franz JR. The age-associated reduction in propulsive power generation in walking. Exercise and Sport Sciences Reviews. 2016;44(4):129-136.

31. 	Chen G, Patten C. Joint moment work during the stance-to-swing transition in hemiparetic subjects. Journal of Biomechanics. 2008;41(4):877-883. doi:10.1016/j.jbiomech.2007.10.017

32. 	Balaban B, Tok F. Gait Disturbances in Patients With Stroke. American Academy of Physical Medicine and Rehabilitation. 2014;6(7):635-642. doi:10.1016/j.pmrj.2013.12.017

33. 	Bowden MG, Balasubramanian CK, Behrman AL, Kautz SA. Validation of a Speed-Based Classification System Using Quantitative Measures of Walking Performance Poststroke. Neurorehabilitation and Neural Repair. 2008;22(6):672-675. doi:10.1177/1545968308318837

34. 	Hsiao H, Awad LN, Palmer JA, Higginson JS, Binder-Macleod SA. Contribution of Paretic and Nonparetic Limb Peak Propulsive Forces to Changes in Walking Speed in Individuals Poststroke. Neurorehabilitation and Neural Repair. 2016;30(8):743-752. doi:10.1177/1545968315624780

35. 	Awad LN, Binder-Macleod SA, Pohlig RT, Reisman DS. Paretic Propulsion and Trailing Limb Angle Are Key Determinants of Long-Distance Walking Function After Stroke. Neurorehabilitation and Neural Repair. 2015;29(6):499-508. doi:10.1177/1545968314554625

36. 	Turns LJ, Neptune RR, Kautz SA. Relationships Between Muscle Activity and Anteroposterior Ground Reaction Forces in Hemiparetic Walking. Archives of Physical Medicine and Rehabilitation. 2007;88(9):1127-1135. doi:10.1016/j.apmr.2007.05.027

37. 	Combs SA, Van Puymbroeck M, Altenburger PA, Miller KK, Dierks TA, Schmid AA. Is walking faster or walking farther more important to persons with chronic stroke? Disability and Rehabilitation. 2013;35(10):860-867. doi:10.3109/09638288.2012.717575

38. 	Neptune RR, Kautz SA, Zajac FE. Contributions of the individual ankle plantar flexors to support, forward progression and swing initiation during walking. Journal of Biomechanics. 2001;34(11):1387-1398. doi:10.1016/S0021-9290(01)00105-1

39. 	Bowden MG, Behrman AL, Neptune RR, Gregory CM, Kautz SA. Locomotor rehabilitation of individuals with chronic stroke: Difference between responders and nonresponders. Archives of Physical Medicine and Rehabilitation. 2013;94(5):856-862. doi:10.1016/j.apmr.2012.11.032

40. 	Fulk GD, He Y, Boyne P, Dunning K. Predicting Home and Community Walking Activity Poststroke. Stroke. 2017;48(2):406-411. doi:10.1161/STROKEAHA.116.015309

41. 	Danks KA, Roos MA, McCoy D, Reisman DS. A step activity monitoring program improves real world walking activity post stroke. Disability and Rehabilitation. 2014;36(26):2233-2236. doi:10.3109/09638288.2014.903303

42. 	Dean CM, Rissel C, Sherrington C, et al. Exercise to Enhance Mobility and Prevent Falls After Stroke: The Community Stroke Club Randomized Trial. Neurorehabilitation and Neural Repair. 2012;26(9):1046-1057. doi:10.1177/1545968312441711

43. 	Dickstein R. Rehabilitation of Gait Speed After Stroke: A Critical Review of Intervention Approaches. Neurorehabilitation and Neural Repair. 2008;22(6):649-660. doi:10.1177/1545968308315997

44. 	Kitago T, Krakauer JW. Motor learning principles for neurorehabilitation. In: Handbook of Clinical Neurology. Vol 110. Elsevier; 2013:93-103. doi:10.1016/B978-0-444-52901-5.00008-3

45. 	van Swigchem R, Roerdink M, Weerdesteyn V, Geurts AC, Daffertshofer A. The Capacity to Restore Steady Gait After a Step Modification Is Reduced in People With Poststroke Foot Drop Using an Ankle-Foot Orthosis. Physical Therapy. 2014;94(5):654-663. doi:10.2522/ptj.20130108

46. 	Vistamehr A, Kautz SA, Neptune RR. The influence of solid ankle-foot-orthoses on forward propulsion and dynamic balance in healthy adults during walking. Clinical Biomechanics. 2014;29(5):583-589. doi:10.1016/j.clinbiomech.2014.02.007

47. 	Wutzke CJ, Sawicki GS, Lewek MD. The influence of a unilateral fixed ankle on metabolic and mechanical demands during walking in unimpaired young adults. Journal of Biomechanics. 2012;45(14):2405-2410. doi:10.1016/j.jbiomech.2012.06.035

48. 	Ellis RG, Howard KC, Kram R. The metabolic and mechanical costs of step time asymmetry in walking. Proceedings of the Royal Society B: Biological Sciences. 2013;280:20122784-20122784. doi:10.1098/rspb.2012.2784

49. 	Halliday SE, Winter DA, Frank JS, Patla AE, Prince F. The initiation of gait in young, elderly, and Parkinson’s disease subjects. Gait & Posture. 1998;8(1):8-14. doi:10.1016/S0966-6362(98)00020-4

50. 	Hass CJ, Buckley TA, Pitsikoulis C, Barthelemy EJ. Progressive resistance training improves gait initiation in individuals with Parkinson’s disease. Gait & Posture. 2012;35(4):669-673. doi:10.1016/j.gaitpost.2011.12.022

51. 	Bethoux F, Bennett S. Evaluating Walking in Patients with Multiple Sclerosis: Which Assessment Tools Are Useful in Clinical Practice? International Journal of MS Care. 2011;13(1):4-14. doi:10.7224/1537-2073-13.1.4

52. 	Franz JR, Kram R. How does age affect leg muscle activity/coactivity during uphill and downhill walking? Gait & Posture. 2013;37(3):378-384. doi:10.1016/j.gaitpost.2012.08.004

53. 	Franz JR, Maletis M, Kram R. Real-time feedback enhances forward propulsion during walking in old adults. Clinical Biomechanics. 2014;29(1):68-74. doi:10.1016/j.clinbiomech.2013.10.018

54. 	Martin M, Shinberg M, Kuchibhatla M, Ray L, Carollo JJ, Schenkman ML. Gait Initiation in Community-Dwelling Adults With Parkinson Disease: Comparison With Older and Younger Adults Without the Disease. Physical Therapy. 2002;82(6):566-577. doi:10.1093/ptj/82.6.566

55. 	Cutting JE, Kozlowski LT. Recognizing friends by their walk: Gait perception without familiarity cues. Bulletin of the Psychonomic Society. 1977;9(5):353-356. doi:10.3758/BF03337021

56. 	Sprager S, Juric M. Inertial Sensor-Based Gait Recognition: A Review. Sensors. 2015;15(9):22089-22127. doi:10.3390/s150922089

57. 	Stanton R, Ada L, Dean CM, Preston E. Biofeedback improves performance in lower limb activities more than usual therapy in people following stroke: a systematic review. Journal of Physiotherapy. 2017;63(1):11-16. doi:10.1016/j.jphys.2016.11.006

58. 	Taborri J, Palermo E, Rossi S, Cappa P. Gait Partitioning Methods: A Systematic Review. Sensors. 2016;16(1):66. doi:10.3390/s16010066

59. 	Dejnabadi H, Jolles BM, Aminian K. A new approach to accurate measurement of uniaxial joint angles based on a combination of accelerometers and gyroscopes. IEEE Transactions on Biomedical Engineering. 2005;52(8):1478-1484. doi:10.1109/TBME.2005.851475

60. 	Donath L, Faude O, Lichtenstein E, Pagenstert G, Nüesch C, Mündermann A. Mobile inertial sensor based gait analysis: Validity and reliability of spatiotemporal gait characteristics in healthy seniors. Gait & Posture. 2016;49:371-374. doi:10.1016/j.gaitpost.2016.07.269

61. 	Evans RL, Arvind DK. Detection of gait phases using orient specks for mobile clinical gait analysis. In: 2014 11th International Conference on Wearable and Implantable Body Sensor Networks. IEEE; 2014:149-154.

62. 	Aurbach M, Wagner K, Süß F, Dendorfer S. Implementation and validation of human kinematics measured using IMUs for musculoskeletal simulations by the evaluation of joint reaction forces. In: CMBEBIH 2017. Springer; 2017:205-211.

63. 	Karatsidis A, Bellusci G, Schepers H, de Zee M, Andersen M, Veltink P. Estimation of Ground Reaction Forces and Moments During Gait Using Only Inertial Motion Capture. Sensors. 2016;17(1):75. doi:10.3390/s17010075

64. 	Ohtaki Y, Sagawa K, Inooka H. A method for gait analysis in a daily living environment by body-mounted instruments. JSME International Journal Series C Mechanical Systems, Machine Elements and Manufacturing. 2001;44(4):1125-1132.

65. 	Guo Y, Storm F, Zhao Y, et al. A New Proxy Measurement Algorithm with Application to the Estimation of Vertical Ground Reaction Forces Using Wearable Sensors. Sensors. 2017;17(10):2181. doi:10.3390/s17102181

66. 	Wouda FJ, Giuberti M, Bellusci G, et al. Estimation of Vertical Ground Reaction Forces and Sagittal Knee Kinematics During Running Using Three Inertial Sensors. Frontiers in Physiology. 2018;9. doi:10.3389/fphys.2018.00218

67. 	Marsland S. Machine Learning: An Algorithmic Perspective. 1st ed. Chapman and Hall/CRC; 2011. doi:10.1201/9781420067194

68. 	Hastie T, Tibshirani R, Friedman J. The Elements of Statistical Learning: Data Mining, Inference, and Prediction. Springer Science & Business Media; 2009.

69. 	Bishop CM. Pattern Recognition and Machine Learning. Springer Science & Business Media; 2006.

70. 	Begg RK, Palaniswami M, Owen B. Support vector machines for automated gait classification. IEEE Transactions on Biomedical Engineering. 2005;52(5):828-838. doi:10.1109/TBME.2005.845241

71. 	Begg R, Kamruzzaman J. A machine learning approach for automated recognition of movement patterns using basic, kinetic and kinematic gait data. Journal of Biomechanics. 2005;38(3):401-408. doi:10.1016/j.jbiomech.2004.05.002

72. 	Mannini A, Trojaniello D, Cereatti A, Sabatini A. A Machine Learning Framework for Gait Classification Using Inertial Sensors: Application to Elderly, Post-Stroke and Huntington’s Disease Patients. Sensors. 2016;16(1):134. doi:10.3390/s16010134

73. 	Tahir NM, Manap HH. Parkinson Disease Gait Classiﬁcation based on Machine Learning Approach. Journal of Applied Sciences. 2012;12:180-185.

74. 	Boutaayamou M, Schwartz C, Stamatakis J, et al. Development and validation of an accelerometer-based method for quantifying gait events. Medical Engineering & Physics. 2015;37(2):226-232. doi:10.1016/j.medengphy.2015.01.001

75. 	Miyazaki T, Kawada M, Nakai Y, Kiyama R, Yone K. Validity of Measurement for Trailing Limb Angle and Propulsion Force during Gait Using a Magnetic Inertial Measurement Unit. BioMed Research International. 2019;2019:1-8. doi:10.1155/2019/8123467

76. 	Peruzzi A, Della Croce U, Cereatti A. Estimation of stride length in level walking using an inertial measurement unit attached to the foot: A validation of the zero velocity assumption during stance. Journal of Biomechanics. 2011;44(10):1991-1994. doi:10.1016/j.jbiomech.2011.04.035

77. 	Seel T, Raisch J, Schauer T. IMU-based joint angle measurement for gait analysis. Sensors. 2014;14(4):6891-6909. doi:10.3390/s140406891

78. 	Yang S, Zhang JT, Novak AC, Brouwer B, Li Q. Estimation of spatio-temporal parameters for post-stroke hemiparetic gait using inertial sensors. Gait & Posture. 2013;37(3):354-358. doi:10.1016/j.gaitpost.2012.07.032

79. 	Karatsidis A, Jung M, Schepers HM, et al. Musculoskeletal model-based inverse dynamic analysis under ambulatory conditions using inertial motion capture. Medical Engineering & Physics. 2019;65:68-77. doi:10.1016/j.medengphy.2018.12.021

80. 	Ryu HX, Park S. Estimation of unmeasured ground reaction force data based on the oscillatory characteristics of the center of mass during human walking. Journal of Biomechanics. 2018;71:135-143. doi:10.1016/j.jbiomech.2018.01.046

81. 	Pieper NL, Lewek MD, Franz JR. Can shank acceleration provide a clinically feasible surrogate for individual limb propulsion during walking? Journal of Biomechanics. Published online October 2019:109449. doi:10.1016/j.jbiomech.2019.109449

82. 	Moore SA, Hickey A, Lord S, Del Din S, Godfrey A, Rochester L. Comprehensive measurement of stroke gait characteristics with a single accelerometer in the laboratory and community: a feasibility, validity and reliability study. Journal of NeuroEngineering and Rehabilitation. 2017;14(1). doi:10.1186/s12984-017-0341-z

83. 	Trojaniello D, Cereatti A, Pelosin E, et al. Estimation of step-by-step spatio-temporal parameters of normal and impaired gait using shank-mounted magneto-inertial sensors: application to elderly, hemiparetic, parkinsonian and choreic gait. Journal of NeuroEngineering and Rehabilitation. 2014;11(1):152. doi:10.1186/1743-0003-11-152

84. 	Mayagoitia RE, Nene AV, Veltink PH. Accelerometer and rate gyroscope measurement of kinematics: an inexpensive alternative to optical motion analysis systems. Journal of Biomechanics. 2002;35(4):537-542. doi:10.1016/S0021-9290(01)00231-7

85. 	Zhang W, Smuck M, Legault C, Ith MA, Muaremi A, Aminian K. Gait Symmetry Assessment with a Low Back 3D Accelerometer in Post-Stroke Patients. Sensors. 2018;18(10):3322. doi:10.3390/s18103322

86. 	Bejarano NC, Ambrosini E, Pedrocchi A, Ferrigno G, Monticone M, Ferrante S. A Novel Adaptive, Real-Time Algorithm to Detect Gait Events From Wearable Sensors. IEEE Transactions on Neural Systems and Rehabilitation Engineering. 2015;23(3):413-422. doi:10.1109/TNSRE.2014.2337914

87. 	Azouvi P, Samuel C, Louis-Dreyfus A, et al. Sensitivity of clinical and behavioural tests of spatial neglect after right hemisphere stroke. Journal of Neurology, Neurosurgery & Psychiatry. 2002;73(2):160-166.

88. 	Plummer P, Behrman AL, Duncan PW, et al. Effects of Stroke Severity and Training Duration on Locomotor Recovery After Stroke: A Pilot Study. Neurorehabilitation and Neural Repair. 2007;21(2):137-151. doi:10.1177/1545968306295559

89. 	Pohl PS. Influence of stroke-related impairments on performance in 6-minute walk test. Journal of Rehabilitation Research and Development. 2002;39(4):7.

90. 	Awad LN, Bae J, O’Donnell K, et al. A soft robotic exosuit improves walking in patients after stroke. Science Translational Medicine. 2017;9(400). doi:10.1126/scitranslmed.aai9084

91. 	Genthe K, Schenck C, Eicholtz S, Zajac-Cox L, Wolf S, Kesar TM. Effects of real-time gait biofeedback on paretic propulsion and gait biomechanics in individuals post-stroke. Topics in Stroke Rehabilitation. 2018;25(3):186-193. doi:10.1080/10749357.2018.1436384

92. 	Kesar TM, Perumal R, Reisman DS, et al. Functional Electrical Stimulation of Ankle Plantarflexor and Dorsiflexor Muscles: Effects on Poststroke Gait. Stroke. 2009;40(12):3821-3827. doi:10.1161/STROKEAHA.109.560375

93. 	McCain EM, Dick TJM, Giest TN, et al. Mechanics and energetics of post-stroke walking aided by a powered ankle exoskeleton with speed-adaptive myoelectric control. Journal of NeuroEngineering and Rehabilitation. 2019;16(1). doi:10.1186/s12984-019-0523-y

94. 	Penke K, Scott K, Sinskey Y, Lewek MD. Propulsive Forces Applied to the Body’s Center of Mass Affect Metabolic Energetics Poststroke. Archives of Physical Medicine and Rehabilitation. 2019;100(6):1068-1075. doi:10.1016/j.apmr.2018.10.010

95. 	Phadke CP. Immediate Effects of a Single Inclined Treadmill Walking Session on Level Ground Walking in Individuals After Stroke: American Journal of Physical Medicine & Rehabilitation. 2012;91(4):337-345. doi:10.1097/PHM.0b013e31823cabe3

96. 	Takahashi KZ, Lewek MD, Sawicki GS. A neuromechanics-based powered ankle exoskeleton to assist walking post-stroke: a feasibility study. Journal of NeuroEngineering and Rehabilitation. 2015;12(1):23. doi:10.1186/s12984-015-0015-7

97. 	Lim H, Kim B, Park S. Prediction of Lower Limb Kinetics and Kinematics during Walking by a Single IMU on the Lower Back Using Machine Learning. Sensors. 2019;20(1):130. doi:10.3390/s20010130

98. 	Shahabpoor E, Pavic A. Estimation of vertical walking ground reaction force in real-life environments using single IMU sensor. Journal of Biomechanics. 2018;79:181-190. doi:10.1016/j.jbiomech.2018.08.015

99. 	Revi DA, Alvarez AM, Walsh CJ, De Rossi SM, Awad LN. Indirect measurement of anterior-posterior ground reaction forces using a minimal set of wearable inertial sensors: From healthy to hemiparetic walking. Journal of neuroengineering and rehabilitation. 2020;17(1):1-13.

100. 	Koo TK, Li MY. A Guideline of Selecting and Reporting Intraclass Correlation Coefficients for Reliability Research. Journal of Chiropractic Medicine. 2016;15(2):155-163. doi:10.1016/j.jcm.2016.02.012

101. 	McGraw KO, Wong SP. Forming inferences about some intraclass correlation coefficients. Psychological Methods. 1996;1(1):30-46. doi:10.1037/1082-989X.1.1.30